### OBJECTIVES:

- To interface a DC motor to the 8051.

- To write a program to control the speed and direction of DC motor rotation by the user.

# REFERENCES:

- Mazidi and McKinlay, “The 8051 Microcontroller and Embedded Systems,” Chapter 17.

### MATERIALS:

- 8051 assembler and trainer

- DC motor

- L293 driver chip

- ILQ74 Opto

### ACTIVITY 1

Using your 8051 trainer, build the circuit shown in Figure 17-18 of the textbook.  Then write and run a program to rotate it clockwise continuously.

### ACTIVITY 2

After making sure that Activity 1 works, write and run the following:

Connect a buffered switch to P2.1 and use it for clockwise or counter clockwise. Use

P2.1=0 for CW and P2.1=1 for CCW.  If you have set up serial 8051 communication, use

the PC to send letters R and L for clockwise and counter clockwise, respectively.

### ACTIVITY 3

After making sure that Activity 1 works, write and run the following:

Use P2.7 and P2.6 to choose the speed of rotation.

Name:						Date: 				Class:

1. What is a use of Opto in Figure 17-18?

2. What is a use of diode in Figure 17-18?

3.  Give the RPM for DC motor used in this Lab

4. What is the disadvantage of using CPL instruction for turning “on” and “off” the DC motor?

5. Explain the PWM