### OBJECTIVES:

- To interface a stepper motor to the 8051.

- To write a program to control the angle and direction of stepper motor rotation by the user.

# REFERENCES:

- Mazidi and McKinlay, “The 8051 Microcontroller and Embedded Systems,” Chapter 17.

- “Design Engineer's Guide to DC Stepping Motor,” Superior Electric Company.

- “The Handbook of Personal Computer Instrumentation,” 5th ed., Burr-Brown, 1990.

### MATERIALS:

- 8051 assembler and trainer

- Stepper motor

- ULN2003 driver chip

### ACTIVITY 1

Using your 8051 trainer, build the circuit shown in Chapter 17 of the textbook.  Then write and run a program to rotate it clockwise continuously. Use the following steps to figure out the stepper motor leads.

1. Use an ohmmeter to measure the resistance of leads. This should identify the COM leads

A through D winding leads.

2. The common wires are connected to the positive side of the motor's power supply. In

many motors, +5V is sufficient.

3. The four leads of the stator winding are controlled by four bits of the 8051 port (in this case

P1.0 - P1.3). However, since the 8051 lacks sufficient current to drive the stepper motor

windings, we must use a driver such as the ULN2003 to energize the stator. Instead of the

ULN2003, we could have used transistors as drivers, as shown in Chapter 17. However,

notice that if transistors are used as drivers, we must also use diodes to take care of

inductive current generated when the coil is turned off. One reason that the ULN2003 is

preferable to the use of transistors as drivers is the fact that the ULN2003 has an internal

diode to take care of back EMF. Change the delay to see the speed of rotation.

### ACTIVITY 2

After making sure that Activity 1 works, write and run a program to rotate clockwise 180 degrees, stop for a second, then go counter clockwise to its original position.

### ACTIVITY 3

After making sure that Activity 1 works, write and run the following:

1. Connect a buffered switch to P2.1 and use it for clockwise or counter clockwise. Use

P2.1=0 for CW and P2.1=1 for CCW.  If you have set up serial 8051 communication, use

the PC to send letters R and L for clockwise and counter clockwise, respectively.

2. After the user has made a choice, the motor will rotate accordingly.

You can also use P2.7 and P2.6 to choose the degree of rotation.

Name:						Date: 				Class:

1. What is a step angle?  Define steps per revolution.

2. If a given stepper motor has a step angle of 5 degrees, find the number of steps per

revolution.

3.  Give the four sequences for counter clockwise if it starts with 10011001 (binary).

4. Using the "RL A" instruction, show the four-step sequences if the initial step is 0011

(binary).

5. Give the number of times the four-step sequence must be applied to a stepper motor to

make a 100-degree move if the motor has a 5-degree step angle. Also fill in the

characteristics for your motor below.

Step angle  ______________	Degree of movement per 4-step sequence  ________

Steps per revolution     _____      Number of rotor teeth  ______________