### OBJECTIVES:

- To examine and use an 8051 assembler.

- To examine and use an 8051 simulator.

### REFERENCE:

- Mazidi and McKinlay, “The 8051 Microcontroller and Embedded Systems,” Chapter 2, Appendix G.

### MATERIALS:

- 8051 assembler and simulator from Franklin Software Inc., Keil Corporation, or the assembler of your choice.

### WEB SITES:

www.fsinc.com 		for Franklin Software

www.keil.com		for Keil Corporation

### ACTIVITY 1

Write and assemble a program to move value 99H to register A. Then from register A move it to all registers R0 - R7. Use the simulator to single-step the program and examine the registers.

### ACTIVITY 2

Write and assemble a program to add all the single digits of your ID number and save the result in R2. Pick 7 random numbers (all single digit) if you do not want to use your ID number. Then use the simulator to single-step the program and examine the registers.

Name:						Date: 				Class:

1.  Indicate the size (8- or 16-bit) of each of the following registers.

PC =			A=			B=

R0=			R1=			R2=		R7=

2. For Question 1, indicate the largest value (in decimal) that each register can contain.

PC =			A=			B=

R0=			R1=			R2=		R7=

3. For Question 1, indicate the largest value (in hex) that each register can contain.

PC =			A=			B=

R0=			R1=			R2=		R7=

4. Who generates each of the following files and what is the use of each.

.asm

.lst

.obj

.abs

.hex