### OBJECTIVES:

- To review memory concepts.

- To examine the memory map of the 8051 on MDE8051 Trainer board

### REFERENCE:

- Mazidi & McKinlay “8051 Microcontroller…..” Chapters 0 1, and 2.

### MATERIALS:

- MDE8051 Trainer

### ACTIVITY 1A

The MDE8051 Trainer uses the Dallas Semiconductor/Maxim 8051 chip called DS89C4x0. Locate the chip datasheet on the www.maxim-ic.com website.

Now, examining the above site we will see that the DS89C4x0 comes with three versions. They are DS89C430, DS89C440, and DS89C450. The DS89C440 is no longer available. You can also locate the memory map of this microcontroller on the http://www.maxim-ic.com/landing/index.mvp?lpk=370

Chip			Flash size

DS89C430  		16K bytes

DS89C440  		32K bytes

DS89C450  		64K bytes

Now, calculate and fill in the following information:

Chip			Flash size	# of Bytes	Memory Address Range

DS89C430  		16K bytes 	……..		……………….

DS89C440  		32K bytes 	……..		……………….

DS89C450  		64K bytes 	……..		……………….

.

Name:						Date: 				Class:

1. Give two differences between EEPROM and Flash memory?

2. Give three differences between SRAM  and Flash memory?

3. For Questions 1 and 2, indicate the use of each memory in microcontroller. In other words, state which one is used for Code, Data, and scratch memory.

4. State the number of address and data pins for 64Kx8 Flash memory.

5. State the number of address and data pins for 4Kx8 SRAM  memory

6. State the number of bytes and K bytes of memory space for the address range of 4000H – 7FFFH (H for Hex)

7. How much on-chip Flash memory do we have in the DS89C440? Give its starting and ending address.

8. State the number of K bytes for address space of 4000H – FFFFH.