### OBJECTIVE:

- To write a program to perform Binary (Hex) to ASCII conversion.

### REFERENCE:

- Mazidi and McKinlay, “The 8051 Microcontroller …,” Chapter 6, 7 and 10.

### MATERIALS:

- 8051 Keil Compiler.

- MDE8051 Trainer

The ADC (Analog to Digital Converter) chips provide output data in binary. To display the data, we must convert them to ASCII.  This Lab shows how to convert an 8-bit binary to ASCII.

Activity 1

Connect the pins of P1 to switches on 8051 Trainer. Then, write a program to convert the binary data on switches to ASCII numbers and display them on HyperTerminal (or Tera Terminal) of the x86 PC.

Name:						Date: 				Class:

- Find the value in registers A and B after the following code.

MOV A,#45

MOV B,#10

DIV	AB

A =       	in hex			in hex

B =       	in hex 			in hex

- Find the value in registers A and B after the following code.

MOV A,#249

MOV B,#10

DIV	AB

A =       	in hex			in hex

B =       	in hex 			in hex

- Find the value in registers A and B after the following code.

MOV A,#101

MOV B,#10

DIV	AB

A =       	in hex			in hex

B =       	in hex 			in hex

- In the 8051, which registers are used for dividing two numbers?

- In the 8051 DIV instruction, what is largest number for numerator?

- In the 8051 DIV instruction, what is largest number for denominator?