### OBJECTIVES:

- To understand the operation modes of an LCD.

- To interface and program an LCD.

### REFERENCES:

- Mazidi and McKinlay, “The 8051 Microcontroller …..,” Chapter 12.

- Dot Matrix LCD Module: Character-type DMC Series User's Manual by Optrex Corp.

### MATERIALS:

- 8051 Keil Compiler

- MDe8051 Trainer

- 20x2 LCD DMC20261 from Optrex DMC series, or a compatible one.

### ACTIVITY 1

Connect the LCD to your 8051 trainer as shown in section 12.1 of the textbook.  Then compile and run the LCD program from the following site:

http://www.microdigitaled.com/8051/Hardware/sample.htm

Now, modify the above program to display you first name and last name, all on the same line..

### ACTIVITY 2

Repeat Activity 1 while also putting the year you graduated from high school on the second line. When you run your program, the LCD should show (for example):

Alex Young

Graduated in 1978

### ACTIVITY 3

Write an LCD program to display your last name on the first line and the current year on the second line. Both should be in the middle of the line.

Name:						Date: 				Class:

1. How does the LCD distinguish data from instruction codes when receiving information at its

data pin?

2.  To send the instruction command code 01 to clear the display, we must make RS = ___.

3.  To send letter 'A' to be displayed on the LCD, we must make RS = ____.

4.  What is the purpose of the E line?  Is it an input or an output as far as the LCD is

concerned?

5.  When is the information (command code or data) on the LCD pin latched into the LCD?