### OBJECTIVE:

- To interface an ADC809 to the 8051.

### REFERENCES:

- Mazidi and McKinlay, “The 8051 Microcontroller …….,” Chapter 13.

- www.national.com/catalog/AnalogDataAquisition.html

### MATERIALS:

- 8051 Keil Compiler

- MDE8051 Trainer

- ADC808/809

- TTL square wave and TTL square wave clock source

In many applications we need to have multiple analog-to-digital converters. The ADC808/809 is a single chip that can accommodate up to 8 analog inputs. These 8 inputs are called channels and each is selected by an address. Read about this chip in Chapter 13 and also download the data sheets from the www.national.com web site. Connect the ADC808/809 to the 8051 Trainer.  For the clock source of the ADC808/809, you can either use an external clock source (square wave TTL), which is preferable, or use the XTAL frequency from the 8051 XTAL as shown in Chapter 13. Make sure that you use at least three or four D flip-flops to bring down the frequency.   Notice the following points when interfacing the 8051 Trainer to the ADC808/809.

1.  The ADC808/809 clock frequency is between 10 kHz and 1250 kHz and must be a

minimum of 3V peak.

2.  The minimum pulse width for ALE = 200 sec and OE = 250 sec.

3.   Read data from the ADC808/809 and follow these steps.

(a)	 Select the channel.

(b)  Activate START (200 sec).

(c)  Poll EOC for high. (you can use time delay instead of polling for this step)

(d)  Send low to OE (for a minimum of 250 sec) and read the data from the ADC.

### ACTIVITY 1

Connect a channel to a potentiometer and LEDs of trainer to P2 pins. Write a program to display data input from a channel on pins of P2. Examine the data displayed on LEDs. As you change the potentiometer, the output should change, indicating the value of the analog input in binary.

### ACTIVITY 2

Notice that the ADC outputs displayed on LEDs is between 0 to FFH where FFH is for a full-scale input. However, this must be converted to decimal and then to ASCII in order to be displayed on the HyperTerminal or LCD. Modify the activity 1 to display the data on LCD or HyperTerminal.

Name:						Date: 				Class:

1.  Indicate the direction of pins ALE, SC, EOC, and OE from the point of view of the

ADC808/809.

2.  Give the steps for converting data and getting the data out of the ADC809. State the status

of the SC and EOC pins in each step.

3.  Give the role of signals ALE, A, B, and C in selecting the ADC channel.

4.  In the ADC809 assume that Vref is connected to 2.56 V. Find the following.

(a) step size

(b) maximum range for Vin

(c) D7 - D0 values if Vin = 1.2 V

(d) Vin if D7 - D0 = 11111111

(e) Vin if D7 - D0 = 10011100

5.  In the ADC809 assume that Vref is connected to 5V. Find the following.

(a) step size

(b) maximum range for Vin

(c) D7 - D0 values if Vin =  2.7 V

(d) Vin  if D7 - D0 = 11111111

(e) Vin  if D7 – D0 = 11011101

6.  In connecting ADC808/809 to an 8051, indicate the direction of pins ALE, SC, EOC,

and OE from the point of view of the 8051.