### OBJECTIVE:

- To access external RAM of DS89C4x0.

### REFERENCES:

- Mazidi and McKinlay, “The 8051 Microcontroller …..,” Chapter 14

### MATERIALS:

- MDE8051 Trainer

- 8051 Keil Compiler

### ACTIVITY 1

Connect the MDE8051 (a DS89C4x0-based) Trainer to the serial port of the x86 PC and a DIP switch to pin P1.x Write and test a program with the following components:

a) if SW=0, the program gets characters from x86 PC keyboard and save them in the external RAM of the DS89C4x0.

b) if SW=1, the program displays the saved data on PC screen.

Name:						Date: 				Class:

1.  Give the following information for the DS89C4x0 external RAM.

Capacity:				Organization:

2.  In the DS89C4x0, give the PMR register address and its value upon reset.