### OBJECTIVES:

- To examine the flag bits of the PSW.

- To examine the stack.

### REFERENCE:

- Mazidi and McKinlay, “The 8051 Microcontroller ……,” Chapter 2 and 3.

### MATERIALS:

- 8051 assembler and simulator.

### ACTIVITY 1

Write and assemble a program to add the following data and then use the simulator to examine the CY flag.

92H, 23H, 66H, 87H, F5H

### ACTIVITY 2

Write and assemble a program to load values into each of registers R0 - R4 and then push each of these registers onto the stack. Single-step the program, and examine the stack and the SP register after the execution of each instruction.

### ACTIVITY 3

Write and assemble a program to:

(a) Set SP = 0D,

(b) Put a different value in each of RAM locations 0D, 0C, 0B, 0A, 09, and 08,

(c) POP each stack location into registers R0 - R4.

Use the simulator to single-step and examine the registers, the stack, and the stack pointer.

Name:						Date: 				Class:

1. Find the value of the CY flag after the execution of  the following code.

(a)	MOV 	A,#85H

ADD	A,#92H

(b)	MOV 	A,#15H

ADD	A,#72H

(c)	MOV 	A,#0F5H

ADD	A,#52H

(d)	MOV A,#0FF

INC A

2. Upon reset, what is the value in the SP register?

3. Upon pushing data onto the stack, the SP register is _____________ (decremented,

incremented).

4. Upon popping data from the stack, the SP register is ___________ (decremented,

incremented).

5. Can you change the value of the SP register?  If yes, explain why you would want

to do that.

6.  The stack uses the same area of RAM as bank ______.