### OBJECTIVES:

- To single step (trace) through a program with a subroutine using a Keil.

### REFERENCE:

- Mazidi and McKinlay “8051 Microcontroller….,” Chapter 3.

### MATERIALS:

- 8051 Keil Assembler/compiler

- 8051 Keil Tutorial

### ACTIVITY 1

After you open a project in Keil copy and paste the Toggle LEDs of P2 program from the http://www.microdigitaled.com/8051/Hardware/sample.htm under MDE8051 Trainer Sample Programs.  See also the tutorial for Keil assembler on the same site.

Now simulate and perform the following steps:

- Single-step through the delay subroutine. Examine the registers of the delay subroutine and make the delay shorter or longer by changing the register values.

- Also examine the contents of the SRAM designated as Stack.

- Simulate the program once more but step over the delay subroutine.

Name:						Date: 				Class:

1. Stack is a section of (SRAM, Flash ROM, EEPROM).

2. True or false. To jump to subroutine the initialization of stack is optional.

3. Stack Pointer register (SP) is …..-bit wide in the 8051.

4. When pushing data onto stack, the SP is (incremented, decremented).

5. Why we use a section of SRAM to define registers R1, R2, …?

6. The PC (program counter) in 8051 is _______-bits wide.

7.  Explain the difference between the following two instructions:

LJMP

SJMP

8.  Explain the difference between the following two instructions:

LCALL

CALL

9. When executing a program  the program counter (PC) is (incremented, decremented).