### OBJECTIVE:

- To input data via 8051 port.

### REFERENCE:

- Mazidi and McKinlay, “The 8051 Microcontroller…….” Chapter 3 and 4

### MATERIALS:

- MDE8051 Trainer

- 8051 Keil Assembler

### ACTIVITY 1

In this Lab we test the 8051's port for input operation. Make sure you do this Lab after you have done Lab4A showing how to download toggle program to MDE8051 Trainer.

Download the Sending P1 Switch Data to P2 LED program from  http://www.microdigitaled.com/8051/Hardware/sample.htm  Then, compile, download and run the program. This program gets data from P1 and sends it to P2. Any change of status of the switches connected to P1 will be instantly reflected on LEDs connected to P2.

### ACTIVITY 2

Write and run a program to get data from P1 and after adding a fixed value of 5 send it to P2.  Set the switches and examine the LEDs and verify the result.

Name:						Date: 				Class:

1. On the 8051 Trainer, which port is closest to the DIP switches?.

2. In this Lab, which port of the 8051 Trainer did we use for inputting data into 8051?  Explain what is role of the first two lines of the program:

MOV A,#0FFH

MOV P1,A

3. On the 8051 Trainer, which port is closest to the LED bar?

4. In this Lab, which port of the 8051 Trainer did we use for outputting?

5. In the 8051, explain why we must write "1" to a port in order for it to be used for input.