### OBJECTIVES:

- To program the 8051 to generate square wave

- To generate a pulse-width-modulation (PWM) square wave.

### REFERENCE:

- Mazidi and McKinlay, “The 8051 Microcontroller …,” Chapters 3 and 4.

### MATERIALS:

- MDE8051 trainer

- 8051 Keil assembler

- Oscilloscope

### ACTIVITY 1

Write a program to generate a square wave frequency on a pin of P2. See the sample program on http://www.microdigitaled.com/8051/Hardware/sample.htm

Now examine the frequency using the oscilloscope. Modify the count value to make it exactly 200Hz frequency.

### ACTIVITY 2

Modify the above program to make Variable frequencies. Connect the 8 DIP switches on the trainer to P1. Then use the Switches to change  the frequency and examine the Freq. oscilloscope. 	If you have access to a Buzzer, connect it to pin P2.7 with a 74LS04 Inverter as a driver and see how the buzzer creates different sounds.

1) Which instruction is used to set a single bit of P2?

2) Which instruction is used to clear a single bit of P2?

3) Which instruction is used to toggle a single bit of P2?

4) Write a short program to create 70% duty cycle pulse on pin P2.4.

5) Write a short program to create 20% duty cycle pulse on pin P2.4.