### OBJECTIVE:

- To write a program to perform BCD to ASCII conversion.

### REFERENCE:

- Mazidi and McKinlay, “The 8051 Microcontroller …,” Chapter 6.

### MATERIALS:

- 8051 Keil assembler .

- MDE8051 Trainer

### Many devices such as GPS and RTC (real timer clock) provide the time and date in packed BCD. To display these data, we must convert them to ASCII.

### ACTIVITY 1

Connect the pins of P1 to DIP switches on 8051 Trainer. Then, write a program to convert the packed BCD data on switches to two ASCII numbers and display them on P2 LEDs with 5 seconds delay in-between each showing.

Name:						Date: 				Class:

- Find the value in A, the accumulator, after the following code.

MOV A,#45H

ANL   A,#0F

A =		in hex

- Find the value in A, the accumulator, after the following code.

MOV A,#45H

ANL  A,#0F0

A =		in hex

3. In the absence of the "SWAP A" instruction, how would you perform the operation?

4. Can the SWAP instruction work on any register?

5. Find the value in A after the following code.

CLR	A

ORL	A,#66H

A =       	in hex

6. Find the value in A after the following code.

CLR	A

CPL	A

ANL	A,#99H

A = 		in hex