### OBJECTIVE:

- To write a program to perform BCD to ASCII conversion.

### REFERENCE:

- Mazidi and McKinlay, “The 8051 Microcontroller …,” Chapter 6 or 7.

### MATERIALS:

- 8051 Keil assembler .

- MDE8051 Trainer

### Binary-to-ASCII conversion is widely used by ADC and sensors.

### ACTIVITY 1

Connect the pins of P1 to DIP switches on 8051 Trainer. Then, Write a program to get a byte of binary data from P1 switches (00-0xFF) and after converting it to ASCII display the result on LEDs of P2 (example 0xFF becomes 323535). Place a 2-second delay in between each of three ASCII showings so you can examine the result.  Run the program to verify it works.

### ACTIVITY 2

Write a program to convert the following ASCII to binary and display the result on LEDs of P2.

ASCII_1	EQU	$31		; Least significant digit

ASCII_2	EQU	$39		; Mid digit

ASCII_3	EQU	$35		; Most significant digit