### OBJECTIVE:

- To create a down-counter.

### REFERENCE:

- Mazidi and McKinlay, “The 8051 Microcontroller ……,” Chapter 7

### MATERIALS:

- MDE 8051 Trainer

- 8051 Keil C Compiler

### In many systems such as microwave oven we use down-counter to keep track of an action. Write a C program to make a down-counter using MDE8051 trainer. It should show the binary count on the LEDs. Change the time delay in between the counts and make sure the time delay is long enough that you can observe the LED counting down.

### ACTIVITY 2

We modify the above program to set the start count to 20 as follows. Change the minimum count to the value of the month you were born and observe the LED counting down to that number.

### ACTIVITY 3

In this activity, use DIP switches to set the start count.

### ACTIVITY 4

Modify your program to count up again to the number set by the DIP SW.