### OBJECTIVES:

- To program the 8051 to generate square wave using C program

- To generate a pulse-width-modulation (PWM) square wave.

### REFERENCE:

- Mazidi and McKinlay, “The 8051 Microcontroller …,” Chapters 7.

### MATERIALS:

- MDE8051 trainer

- 8051 Keil C Compiler

- Oscilloscope

### ACTIVITY 1

Write a C program to generate a square wave frequency on a pin of P2. See the sample program on http://www.microdigitaled.com/8051/Hardware/sample.htm

Now examine the frequency using the oscilloscope. Modify the count value to make it exactly 200Hz frequency.

### ACTIVITY 2

Modify the above program to make Variable frequencies. Connect the 8 DIP switches on the trainer to P1. Then use the Switches to change the frequency and examine the Freq. oscilloscope. 	If you have access to a Buzzer, connect it to pin P2.7 with a 74LS04 Inverter as a driver and see how the buzzer creates different sounds.