### OBJECTIVES:

- To program the 8051 timer.

- To generate a square wave using the 8051 timer and play music.

### REFERENCE:

- Mazidi and McKinlay, “The 8051 Microcontroller ….,” Chapter 9.

### MATERIALS:

- MDE8051 Trainer

- 8051 Keil Compiler

- Oscilloscope

- Speaker (optional)

- 74LS04 Inverter (optional)

### ACTIVITY 1

In LED Toggle Lab we used time delay based on NOP instruction. One can use the 8051 Timer to generate time delay. Download the Toggling bit Using Timer program from

http://www.microdigitaled.com/8051/Hardware/sample.htm

Compile and run it on the MDE8051 trainer to make sure it works.

### ACTIVITY 2

Write a program using a Timer to generate a 500 Hz square wave frequency on pin P2.7 Then examine the frequency using the oscilloscope.  Modify the count value to make sure that the frequency is exactly 500 Hz.

### ACTIVITY 3

Write a program using a Timer to generate a 1 kHz square wave frequency on pin of P2.0 Then examine the frequency using the oscilloscope. Modify the count value to make sure that the frequency is exactly 1 kHz.

### ACTIVITY (Advanced)

Connect the P2.7 to a speaker as shown below. Also connect the DIP SW of the Trainer to P1.7 and write a program to generate various notes of music. Use the DIP switches to change the frequency for notes. The frequency for various notes are shown on the next page.

Name:						Date: 				Class:

1. What is the maximum frequency that can be generated using Mode 1 if the crystal frequency

is 11.0592 MHz?  Show your calculation.

2. What is the maximum frequency that can be generated using Mode 2 if the crystal

frequency is 11.0592 MHz?  Show your calculation.

3. What is the lowest frequency that can be generated using Mode 1 if the crystal frequency is

11.0592 MHz?  Show your calculation.

4. What is the lowest frequency that can be generated using Mode 1 if the crystal frequency is

11.0592 MHz? Show your calculation.

5. In mode 1, when is TFx set to high?

6. In mode 2, when is TFx set to high?