### OBJECTIVES:

- To program the 8051 timer/counter as an event counter.

.

### REFERENCE:

- Mazidi and McKinlay, “The 8051 Microcontroller …….,” Chapter 9.

### MATERIALS:

- MDE8051 Trainer

- 8051 Keil Compiler

- Square wave generator

For this lab you need an external source of the square wave. This can be from a signal generator (or some call it a function generator).

### ACTIVITY 1

Connect the square wave from function generator to pin P3.4 for Timer 0 and set the frequency to 1 Hz. Write and run a program for counter/timer 0 to count up from 00 to a maximum of 255 (FFH) as each pulse is fed into the 8051. The count is displayed in binary on the 8 LEDS of 8051 Trainer.  Notice that the binary numbers displayed for the count goes from 00 to FF (00000000- 11111111 in binary or 00 -255 in decimal).

Name:						Date: 				Class:

1. Explain the role of the C/T bit in the TMOD register.

2. How is the 8051 used as an event counter to count an external event?

3. If timer/counter 0 is used as an event counter, what is the maximum count for the following

modes?

Mode 1

Mode 2

4. Indicate which pin is used for the following.

(a) timer/counter 0

(b) timer/counter 1

5. If timer/counter 0 is used in mode 1 to count an external event, explain when TF0 is set to

high.

6. If timer/counter 1 is used in mode 2 to count an external event, explain when TF0 is set to

high.