### OBJECTIVES:

- To interface a Centronics printer to the 8051.

- To gain an understanding of printer handshaking signals.

### REFERENCE:

- Mazidi and Mazidi, “The 8051 Microcontroller and Embedded Systems,” Chapter 15, plus Figure C-20.

### MATERIALS:

- 8051 assembler and trainer

- Centronics printer

- 74LS244

### ACTIVITY 1

Examine the Centronics cable pins and their description in Chapter 15 of the textbook. Connect the data bus and handshaking signals of the printer to your 8051 as shown in Figure C-20. Make sure you have buffered the signals with 74LS244 chips. Write and run a program to print the message, "OK IT WORKS !"

### ACTIVITY 2

Make the following modifications in the program of Activity 1. In the code section put the following data:

Last name, First name

Date

Course number

Each of the above must be printed on a separate line.

Use the ASCII code OA (hex) for line feed and OD (hex) for carriage return to make sure that each line is printed on a separate line, as shown below:

ORG 200

MYDATA:	DB	"Smart Alex",OAH,0DH

Name:						Date: 				Class:

1.  Indicate the direction of the following signals from the printer's point of view.

Signal         		Direction

D0-D7

STROBE

ACK

2. Indicate the direction of the following signals from the 8051's point of view.

Signal         		Direction

D0-D7

STROBE

ACK

- For each of the following signals, indicate its normal status (high or low) and the

activation level for each (active low or active high).

Printer's Handshake Signal    		Normal State    	Active State

Strobe

ACK

4. Explain the role of the strobe and busy signals in the handshaking between the 8051 and the

printer.

5. To make the printer latch in data at its data pins, the strobe pins needs a ______________

(H-to-L, L-to-H) signal.

6. In monitoring the busy signal, the 8051 looks for _______ (high, low) before issuing the

next byte of data to the printer.