### OBJECTIVES:

- To program the 8051 timer.

- To generate a square wave using the 8051 timer.

### REFERENCE:

- Mazidi and Mazidi, “The 8051 Microcontroller and Embedded Systems,” Chapter 9.

### MATERIALS:

- 8051 trainer

- 8051 assembler

- Oscilloscope

### ACTIVITY 1

Write a program using timer 0 to generate a 500 Hz square wave frequency on one of the pins of P1. Then examine the frequency using the oscilloscope.  Modify the count value to make sure that the frequency is exactly 500 Hz.

### ACTIVITY 2

Write a program using timer 1 to generate a 1 kHz square wave frequency on one of the pins of P1. Then examine the frequency using the oscilloscope. Modify the count value to make sure that the frequency is exactly 1 kHz.

Name:						Date: 				Class:

1. What is the maximum frequency that can be generated using Mode 1 if the crystal frequency

is 11.0592 MHz?  Show your calculation.

2. What is the maximum frequency that can be generated using Mode 2 if the crystal

frequency is 11.0592 MHz?  Show your calculation.

3. What is the lowest frequency that can be generated using Mode 1 if the crystal frequency is

11.0592 MHz?  Show your calculation.

4. What is the lowest frequency that can be generated using Mode 1 if the crystal frequency is

11.0592 MHz? Show your calculation.

5. In mode 1, when is TFx set to high?

6. In mode 2, when is TFx set to high?