### OBJECTIVES:

- To examine the I/O port operation using a simulator.

- To trace through a CALL subroutine using a simulator.

### REFERENCE:

- Mazidi and Mazidi, “The 8051 Microcontroller and Embedded Systems,” Chapters 3 and 4.

### MATERIALS:

- 8051 assembler and simulator.

### ACTIVITY 1

Write and assemble a program to toggle all the bits of P0, P1, and P2 continuously by sending 55H and AAH to these ports. Put a time delay between the "on" and "off" states.  Then using the simulator, single-step through the program and examine the ports. Do not single-step through the time delay call.

### ACTIVITY 2

Using a simulator, single-step the program in Activity 1. Single-step through the delay subroutine. Examine the registers of the delay subroutine and make the delay shorter or longer by changing the register values.

### ACTIVITY 3

Using a simulator, write a program to get a byte of data from P1 and send it to P0 and P2. Also, give a copy of it to registers R0, R1, and R2. Single-step the program and examine the ports and registers.

Name:						Date: 				Class:

1. Upon reset, all the ports of the 8051 are configured as ______ (input, output).

2. To make all the bits of a port an input port we must write ____ hex to it.

3. Which ports of the 8051 are bits addressable?

4. What does it mean for port to be "read-modify-write"?

5. Write a program to monitor P2.4 continuously. When it becomes low, it sends 55H to P1.