### OBJECTIVE:

- To write a program to perform ASCII and BCD conversion.

### REFERENCE:

- Mazidi and Mazidi, “The 8051 Microcontroller and Embedded Systems,” Chapter 6 or 7.

### MATERIALS:

- 8051 assembler/C compiler and simulator.

### ACTIVITY 1

Write a program to convert the following ASCII data to packed BCD.

ORG	120H

MYDATA:	DB	'1998'

Place the packed BCD result in RAM locations starting at 40H. Using a simulator, single-step the program and examine the data.

### ACTIVITY 2

Write a program to convert the following BCD data to ASCII. Place the result in RAM locations starting at 50H. The BCD data is stored in ROM starting at 150H. The data is stored as follows:

ORG	150H

MYDATA:	DB	42H,58H,64H,29H		;pick your own data.

Using a simulator, single-step the program and examine the data.

### ACTIVITY 3

Write a program to find the average of the following numbers. Notice that they are ASCII and must be converted to BCD before the average is calculated.

ORG 120H

MYDATA:	DB	'8','9','4','7','6','2','5'	;pick your own data

Notice that you must first bring the data from ROM space into the CPU's RAM, then add them together. Using a simulator, single-step the program and examine the data.

Name:						Date: 				Class:

- Find the value in A, the accumulator, after the following code.

MOV A,#45H

RR	A

RR	A

RR	A

A =		in hex

- Find the value in A, the accumulator, after the following code.

MOV A,#45H

RL	A

RL	A

RL	A

A =		in hex

3. In the absence of the "SWAP A" instruction, how would you perform the operation?

4. Can the SWAP instruction work on any register?

5. Find the value in A after the following code.

CLR	A

XRL	A,#0FFH

A =       	in hex

6. Find the value in A after the following code.

CLR	A

CPL	A

XRL	A,#0FFH

A = 		in hex