# OBJECTIVES:

- To program the timer interrupt of the AVR.

- To program the hardware external interrupt of the AVR.

# REFERENCES:

- Mazidi and Naimi, “The AVR Microcontroller and Embedded Systems,” Chapter 10.

# MATERIALS:

- AVR Trainer

- AVR Studio

- Square wave generator

# ACTIVITY 1

- Write a program to get data from port B and send it to port C continuously while an interrupt will do the following: One of the timers will toggle pin PA2 every 100 microseconds.  In place of port B, port C, and PA2 pin you can use ports of your own choice.

# ACTIVITY 2

- Add the following interrupt to Activity 1: A square wave is connected to external interrupt and any time a L-to-H pulse comes in, a single LED is turned on, and any time a H-to-L pulse comes in, the LED is turn off. The rate of "On" and "Off" is the same as the square wave frequency.

# ACTIVITY 3

- Add the following interrupt to Activity 2: Connect one of the external interrupt pins to a normally high debounce SW and any time the switch is pressed, a single LED is toggled. The rate of "On" and "Off" is half of the square wave frequency.

- For ATmega32, name all the interrupts associated with hardware and timers.

- Indicate the usage of timer interrupt.

- On reset, external interrupt 0 is _________ (edge, level, change) triggered.

- True or False. On reset, all interrupts are active.