# OBJECTIVES:

- To understand the operation modes of an LCD.

- To interface and program an LCD.

# REFERENCES:

- Mazidi and Naimi, “The AVR Microcontroller and Embedded Systems,” Chapter 12.

- Dot Matrix LCD Module: Character-type DMC Series User's Manual by Optrex Corp.

# MATERIALS:

- AVR Assembler

- AVR Trainer

- 20x2 LCD DMC20261 from Optrex DMC series, or a compatible one.

# ACTIVITY 1

Connect the LCD to your AVR trainer as shown in Section 12.1 of the textbook. Then write and run a program to display your name on line 1 of the LCD (first name followed by last name with a space in between).

Note: If you are not monitoring the busy flag of the LCD, put a few milliseconds delay in your program. See the discussion in Chapter 12.

# ACTIVITY 2

Repeat Activity 1 while also putting the year you graduated from high school on the second line. When you run your program, the LCD should show (for example):

Alex Young Graduated in 1978

# ACTIVITY 3

Write an LCD program to display your last name on the first line and the current year on the second line. Both should be in the middle of the line.

- How does the LCD distinguish data from instruction codes when receiving information at its data pin?

- To send the instruction code 01 to clear the display, we must make RS = _____.

- To send letter 'A' to be displayed on the LCD, we must make RS = ______.

- What is the purpose of the E line?  Is it an input or an output as far as the LCD is concerned?

- When is the information (code or data) on the LCD pin latched into the LCD?