# OBJECTIVEs:

- To interface a DC motor to AVR.

- To write a program to control the speed and direction of DC motor rotation by the user.

# REFERENCE:

- Mazidi and Naimi “The AVR Microcontroller and Embedded System,” Chapter 16.

# MATERIAL:

- AVR Studio

- AVR Trainer

- DC motor module

Connect your AVR Trainer to the DC motor module or build the circuit shown in chapter 16.  (Figure 16-7 or Figure 16-8)

# ACTIVITY 1

Write and run a program to rotate the DC motor clockwise continuously.

# ACTIVITY 2

After making sure that Activity 1 works, connect two switches to one of the ports and write and run the following program:

Use the SW1 and SW2 to choose the speed of rotation, as shown in the following table.

| SW2 | SW1 | Duty Cycle |
| --- | --- | --- |
| 0 | 0 | 25% |
| 0 | 1 | 50% |
| 1 | 0 | 75% |
| 1 | 1 | 100% |

# ACTIVITY 3

Connect a switch to a pin on one of the PORTs. Use this SW for the purpose of choosing clockwise (CW) or counter clockwise (CCW) direction. Write and run a program to rotate the motor as follows:

SW=0 for CW.

SW=1 for CCW.

- What is the use of Opto-isolator in Figure 16-8?  Does your DC module have one?

- In Activity 1, explain how you turned the DC motor “on” and “off”.

- In Activity 2, explain how you controlled the speed of DC motor.

- Explain the PWM

- What is the disadvantage of using a group of instructions (instead of PWM feature of AVR) in controlling the speed of the DC motor?