# OBJECTIVES:

- To examine the flag bits of the SREG.

# REFERENCES:

- Mazidi and Naimi, “The AVR Microcontroller and Embedded Systems,” Chapter 2.

# MATERIALS:

- AVR Studio.

# ACTIVITY 1

- Write and assemble a program to add the following data and then use the simulator to examine the C, H and Z flags after the execution of each addition.

- $92, $23, $66, $87, $F5

# ACTIVITY 2

- Write and assemble the following program. Use the simulator to single-step and examine the flags and register content after the execution of each instruction.

.INCLUDE "M32DEF.INC"

LDI	R20,$27

LDI	R21,$15

SUB	R20, R21

LDI	R20,$20

LDI	R21,$15

SUB	R20, R21

LDI	R24,95

LDI	R25,95

SUB	R24, R25

LDI	R22,50

LDI	R23,70

SUB	R22, R23

L1:	RJMP	L1

# ACTIVITY 3

- Write and assemble a program to load a value into each location of R20 – R23. Use the COM instruction to complement the value in each register. Use the simulator to single-step and examine the flags and register content after the execution of each instruction.

.INCLUDE "M32DEF.INC"

LDI	R20,0

LDI	R21,0xFF

LDI	R22,0x11

LDI	R23,0x22

COM	R20

COM	R21

COM	R22

COM	R23

L1:	RJMP	L1

- Find the value of the C flag after the execution of the following codes.

- (a)	LDI 	R20, $85

- LDI	R21, $92

- ADD	R20, R21

- (b)	LDI 	R16, $15

- LDI	R17, $72

- ADD	R16, R17

- (c)	LDI 	R25, $F5

- LDI	R26, $52

- ADD	R25, R26

- (d)	LDI 	R25, $FE

- INC 	R25

- INC 	R25