# OBJECTIVES:

- To examine the stack.

# REFERENCE:

- Mazidi and Naimi, “The AVR Microcontroller and Embedded Systems”, Chapter 3.

# MATERIALS:

- AVR Studio

# ACTIVITY 1

Write and assemble a program to load values into each of registers R20 – R24 and then push each of these registers onto the stack. Single-step the program, and examine the stack and the SP register after the execution of each instruction.

.INCLUDE "M32DEF.INC"

LDI	R20, HIGH(RAMEND)

OUT	SPH, R20

LDI	R20, LOW(RAMEND)

OUT	SPL, R20

LDI	R20, $20

LDI	R21, $31

LDI	R22, $42

LDI	R23, $53

LDI	R24, $64

PUSH	R20

PUSH	R21

PUSH	R22

PUSH	R23

PUSH	R24

L1:	RJMP	L1

# ACTIVITY 2

- Write and assemble a program to:

- Set SP = $9D,

- Put a different value in each of RAM locations $9D, $9C, $9B, $9A, $99, and $98,

- POP each stack location into registers R20 – R24.

- Use the simulator to single-step and examine the registers, the stack, and the stack pointer.

.INCLUDE "M32DEF.INC"

LDI	R20, HIGH($9D)

OUT	SPH, R20

LDI	R20, LOW($9D)

OUT	SPL, R20

LDI	R16, 1

STS	$98, R16

LDI	R16, 2

STS	$99, R16

LDI	R16, 3

STS	$9A, R16

LDI	R16, 4

STS	$9B, R16

LDI	R16, 5

STS	$9C, R16

LDI	R16, 6

STS	$9D, R16

POP	R20

POP	R21

POP	R22

POP	R23

POP	R24

L1:	RJMP	L1

- Upon reset, what is the value in the SP register?

- Upon pushing data onto the stack, the SP register is _____________ (decremented,   incremented).

- Upon popping data from the stack, the SP register is ___________ (decremented, incremented).

- Can you change the value of the SP register?  If yes, explain why you would want to do that.