# OBJECTIVES:

- To examine the I/O port operation using a simulator.

- To trace through a CALL subroutine using a simulator.

# REFERENCE:

- Mazidi and Naimi, “The AVR Microcontroller and Embedded Systems,” Chapters 3 and 4.

# MATERIALS:

- AVR Studio

# ACTIVITY 1

Write and assemble a program to toggle all the bits of PORTA, PORTB, and PORTC continuously by sending $55 and $AA to these ports. Put a time delay between the "on" and "off" states.  Then using the simulator, single-step through the program and examine the ports. Do not single-step through the time delay call.

# ACTIVITY 2

Using a simulator, single-step the program in Activity 1. Single-step through the delay subroutine. Examine the registers of the delay subroutine and make the delay shorter or longer by changing the register values.

# ACTIVITY 3

- Using a simulator, write a program to get a byte of data from PORTB and send it to PORTA and PORTC. Also, give a copy of it to registers R20, R21, and R22. Single-step the program and examine the ports and registers.

- Upon reset, all the ports of the AVR are configured as ______ (input, output).

- To make all the bits of a port an input port we must write ____ hex to DDRx.

- Which ports of the AVR are bits addressable?

- Write a program to monitor port B.0 continuously. When it becomes low, it sends $55 to PORTA.