# OBJECTIVE:

- To perform I/O operation on the AVR port.

# REFERENCE:

- Mazidi and Naimi, “The AVR Microcontroller and Embedded System,” Chapters 3 and 4

# MATERIALS:

- AVR Trainer

- AVR Assembler

# ACTIVITY 1

- Test the AVR’s ports for input operation as follows.

- Connect each pin of PORTx (PORTx.0 to PORTx.7) of the AVR to DIP switches of AVR Trainer. Also connect each pin of PORTy to an LED on the LED bar of the AVR Trainer.

- Then, write and run a program to get data from PORTx and send it to PORTy. Any change of status of the switches connected to PORTx will be instantly reflected on LEDs connected to PORTy. The testing program could look like this.

.INCLUDE "M32DEF.INC"

LDI	R20, 0x00	;make port A an input port

OUT	DDRA, R20

LDI	R20, 0xFF	;make port B an output port

OUT	DDRB, R20

OUT	PORTA, R20	;enable pull-up resistors

L1:	IN	R20,PINA	;get data from port A

OUT	PORTB,R20	;send it to port B

RJMP	L1		;keep doing this

# ACTIVITY 2

- Write and run a program to get data from PORTx and after adding a fixed value of 5 send it to PORTy.

- Set the switches and examine the LEDs and verify the result.

.INCLUDE "M32DEF.INC"

LDI	R20, 0x00	;make port A an input port

OUT	DDRA, R20

LDI	R20, 0xFF	;make port B an output port

OUT	DDRB, R20

OUT	PORTA, R20	;enable pull-up resistors

L1:	IN	R20,PINA	;get data from port A

ADD	R20, 5

OUT	PORTB,R20	;send it to port B

RJMP	L1		;keep doing this

- On the AVR Trainer, which port is closest to the DIP switches?

- In this Lab, which port of the AVR Trainer did we use for inputting data into AVR?  Explain what is role of lines 2 and 3 of the program:

LDI	R20, 0x00

OUT	DDRA, R20

- On the AVR Trainer, which port is closest to the LED bar?

- In this Lab, which port of the AVR Trainer did we use for outputting?

- In Activity 2, give the highest number (in decimal) we can have on switches without losing the carry bit.