# OBJECTIVE:

- To write a program to perform calculation.

# REFERENCE:

- Mazidi and Naimi, “The AVR Microcontroller and Embedded Systems,” Chapter 5.

# MATERIALS:

- AVR Studio

# ACTIVITY 1

- Write a program that calculates (PORTA + 4) * PORTB and sends out the result through PORTC and PORTD. Consider all the values unsigned.

# ACTIVITY 2

Write a program to calculate the result of (PORTA + PORTB)/2 and sends out the result through PORTC. Consider all the values unsigned.

# ACTIVITY 3

Write a program that calculates (PORTA + 4) * PORTB and sends out the result through PORTC and PORTD. Consider all the values signed.

- Find the value in R0 and R1 after the following code.

LDI 	R16, 10

LDI 	R17, 20

LDI 	R18, 30

MUL	R16, R17

ADD	R0, R18

R0 =		in hex

R1 =		in hex

- Find the value in R0 and R1 after the following code.

LDI 	R19, 19

SUBI	R19, 10

LDI	R30, 30

MUL	R30, R19

R0 =		in hex

R1 =		in hex