# OBJECTIVES:

- To code a program to transfer data from program memory into RAM locations.

- To code a program to transfer data from RAM locations to other RAM locations.

- To experiment with a look-up table.

# REFERENCE:

- Mazidi and Naimi, “The AVR Microcontroller and Embedded Systems,” Chapter 6.

# MATERIALS:

- AVR Studio

# ACTIVITY 1

- Write a program to transfer a string of data from program memory starting at address $200 to RAM locations inside the CPU starting at $140. The data representing your last name and first name is as shown below:

MYDATA:	.DB	"Alex Young", 0

Using the simulator, single-step through the program and examine the data transfer and registers.

# ACTIVITY 2

- Add the following subroutine to the program in Activity 1, single-step through the subroutine and examine the RAM locations:

- After data has been transferred from program memory into RAM, the subroutine should copy the data from RAM locations starting at $140 to RAM locations starting at $160.

# ACTIVITY 3

- Write a program to calculate y where y = x2 + 2x + 9. x is between 0 and 9 and the look-up table for x2 is located at the address (program memory) of $200. Register R20 has the x, and at the end of the program R21 should have y. Use the simulator to change the x value and single-step through the program, examining the registers as you go.

- Explain the difference between the following two instructions:

  - LPM 	R16, Z

  - LD	R16, Z

- Circle the invalid instructions.

  - LDS	R20, 60

  - LD	R30, Z

  - LD	R25, Z+

  - LPM	R25, Z+4

- Explain the difference between the following two instructions:

  - LDS	R20, $40

  - LDI 	R20, $40