# OBJECTIVE:

- To interface seven-segment to the AVR trainer.

# REFERENCES:

- Mazidi and Naimi, “The AVR Microcontroller and Embedded Systems,” Chapters 4, 6, and page 615.

# MATERIALS:

- AVR trainer

- AVR assembler

- Two 7-segments

# ACTIVITY 1

- Connect a common cathode 7-segment directly to PORTA.

- Write the following program in the AVR Studio and then build and download it to the trainer board. This program displays 7 on the 7-segment.

INCLUDE "M32DEF.INC"

LDI	R20, 0xFF

OUT	DDRA, R20

LDI	R20, 0b00000111

OUT	PORTA, R20

HERE: RJMP	HERE

# ACTIVITY 2

Modify the previous program to display 9 on the 7-segment.

# ACTIVITY 3

- Using look-up table write a subroutine that displays the contents of R21 on the 7-segment.

# ACTIVITY 4 (Advanced)

- Now connect two 7-segments to the same port of AVR, as shown below.

- Display 23 by scanning on the 7-segments.

…

L1:	LDI	R20,2

CALL	DISPLAY_ON_7SEGMENT

LDI	R16,0x01

OUT	PORTB, R16

CALL	DELAY_10ms		;wait for a while

LDI	R20,3

CALL	DISPLAY_ON_7SEGMENT

LDI	R16,0x02

OUT	PORTB, R16

CALL	DELAY_10ms		;wait for a while

RJMP L1