# OBJECTIVES:

- To code a program to add hex numbers.

- To code a program to add BCD numbers.

- To code a program to add two multi-byte BCD numbers.

- To practice converting data from decimal to binary and hexadecimal systems.

# REFERENCE:

- Mazidi and Naimi, “The AVR Microcontroller and Embedded Systems,” Chapters 5 and 6.

# MATERIALS:

- AVR Studio

# ACTIVITY 1

- Write a program to add 10 bytes of data and store the result in registers R30 and R31. The bytes are stored in the Program memory starting at $200. The data would look as follows:

MYDATA:	.DB	92,34,84,129,...		;pick your own data.

- Notice that you must first bring the data from Program memory into the registers, then, add them together. Use a simulator to single-step the program and examine the data.

# ACTIVITY 2

- Write a program to add 10 bytes of BCD data and store the result in R30 and R31. The bytes are stored in ROM space starting at $300. The data would look as follows:

MYDATA:	.DB	$92,$34,$84,$29,...	;pick your own data.

- Notice that you must first bring the data from Program memory into the registers, then, add them together. Use a simulator to single-step the program and examine the data.

# ACTIVITY 3

- Write a program to add two multi-byte BCD numbers together and store the result in RAM locations $100 - $104. The two multi-byte items are stored in the program memory starting at $120 and $150. See the following example data.

.ORG  $120

DATA_1:		.DB	$54,$76,$65,$98	;number 98657654

DATA_2:		.DB	$93,$56,$77,$38	;number 38775693

- Notice that you must first bring the data from Program memory into the registers, then, add them together. Use a simulator to single-step the program and examine the data.

- Explain the difference between the ADD and ADC instructions.

- Show how to perform the subtraction: $29 - $21.

- Show a simple program to add $2345 and $56F8

- Show a simple program to subtract $2345 from $56F8.