# OBJECTIVE:

- To write a program to perform bit manipulations.

# REFERENCE:

- Mazidi and Naimi, “The AVR Microcontroller and Embedded System,” Chapters 5 and 6

# MATERIALS:

- AVR Studio

# ACTIVITY 1

Write a program that copies bits 1, 2, and 3 of PINB to bits 0, 1, and 2 of port C.

# ACTIVITY 2

Write a program that copies bits 1 and 6 of PINB to bits 0, and 2 of port C, respectively.

# ACTIVITY 3

Write a program that inverts bit 3 of port A and sends it to bit 0 of port C.

- Find the value in R16 after the following code.

LDI 	R16, $45

ROR	R16

ROR	R16

ROR	R16

R16 =		in hex

- Find the value in R16 after the following code.

LDI 	R16, $45

ROL	R16

ROL	R16

ROL	R16

R16 =		in hex

- In the absence of the "SWAP Rn" instruction, how would you perform the operation?

- Can the SWAP instruction work on any register?

- Find the value in R2 after the following code.

CLR	R2

LDI	R21, $FF

EOR	R2, R21

R2 =       	in hex

- Find the value in A after the following code.

CLR	R10

COM	R10

LDI	R16, $AA

EOR	R10, R16

R10 = 		in hex