# OBJECTIVE:

- To access EEPROM of AVR

# REFERENCES:

- Mazidi and Naimi, “The AVR Microcontroller and Embedded Systems,” Chapters 6 and 11.

# MATERIALS:

- AVR Studio

- AVR Trainer

# ACTIVITY 1

- Connect a Switch to a pin on one of the PORTs and an LED to another pin. Write and test a program with the following components:

- The LED toggles when the switch is pressed.

- Store the last state of the LED in the EEPROM so that, if the microcontroller powers off, it restores its last state when it powers on, again.

# ACTIVITY 2

- Connect the AVR Trainer to the Serial Port of the x86 IBM PC and a Switch to a pin on one of the PORTs. Write and test a program with the following components:

- if SW=0 we will have the following:

  - The AVR Trainer sends a message to the PC’s Hyperterminal asking the user for a string of characters to be typed.

  - The program gets the characters from x86 PC keyboard and saves them in an internal buffer in the RAM.

  - The AVR will write the contents of the RAM buffer to EEPROM of the AVR.

2) if SW=1, the program displays the data saved in EEPROM on PC screen.

- Give the following information about the EEPROM of the AVR used in Lab.

Capacity:				Organization:

- In the AVR, give the registers used for writing data to EEPROM.  Explain the role each register plays in the process.

- In the AVR, give all the registers used for reading data from EEPROM. Explain the role each register plays in the process.

- Compare and contrast the Flash and EEPROM memories in the AVR microcontroller.

- Compare and contrast the process of reading from the Flash and EEPROM memories in the AVR microcontroller.