# OBJECTIVE:

- To write a program to perform ASCII and BCD conversion.

# REFERENCE:

- Mazidi and Naimi, “The AVR Microcontroller and Embedded Systems,” Chapter 6.

# MATERIALS:

- AVR Studio

# ACTIVITY 1

- Write a program to convert the following ASCII data to packed BCD.

.ORG	$120

MYDATA:	.DB	'1998'

- Place the packed BCD result in RAM locations starting at $100. Using a simulator, single-step the program and examine the data.

# ACTIVITY 2

- Write a program to convert the following BCD data to ASCII. Place the result in RAM locations starting at $150. The BCD data is stored in program memory starting at $150. The data is stored as follows:

.ORG	$150

MYDATA:	.DB	$42,$58,$64,$29		;pick your own data.

- Using a simulator, single-step the program and examine the data.

# ACTIVITY 3

- Write a program to find the average of the following numbers. Notice that they are ASCII and must be converted to BCD (or binary) before the average is calculated.

.ORG $120

MYDATA:	.DB	'8','9','4','7','6','2','5'	;pick your own data

- Notice that you must first bring the data from program memory into registers, then, add them together. Using a simulator, single-step the program and examine the data.

- To examine the I/O port operation using a simulator.

- To trace through a CALL subroutine using a simulator.

- Write a program that loads R16 with 45 in BCD.

- Register R20 contains $25. What is its content if we consider it as a BCD number? What if we consider it as a binary number? What if we consider it as an ASCII character? (see Appendix F)