# OBJECTIVES:

- To program the AVR timer.

- To generate a square wave using the AVR timer and play music.

# REFERENCE:

- Mazidi & Naimi, “The AVR Microcontroller and Embedded Systems,” Chapter 9.

# MATERIALS:

- AVR Trainer

- AVR Assembler

- Oscilloscope

- Speaker (optional)

# ACTIVITY 1

- Write a program using a Timer to generate a 500 Hz square wave frequency on pin PORTC.7.

- Then examine the frequency using the oscilloscope.

- Modify the count value to make sure that the frequency is exactly 500 Hz.

#include "avr/io.h"

void delay ( )

{

TCNT0 = -125;

TCCR0 = 0x3;

while ((TIFR&0x01) == 0);

TIFR = 0x01;

}

int main ( )

{

DDRC = 0x80;

while(1)

{

PORTC = 0x80;

delay ( );

PORTC = 0x00;

delay ( );

}

return 0;

}

# ACTIVITY 2

- Modify the Activity 1 to generate a 1000 Hz square wave frequency.

- Rewrite the code, using the CTC mode.

# ACTIVITY 3

- Using Timer1, write a program that sets the PORTA.2 when the key is pressed and after 3 seconds PORTA.3 is turned on, as well.

# ACTIVITY 4

- Connect a key to PORTB.0. Write a program to toggle PORTB, when the key is pressed, 10 times.

- What is the lowest frequency that can be generated using Timer0 if the crystal frequency is 8 MHz?  Show your calculation.

- What is the lowest frequency that can be generated using Timer1 if the crystal frequency is 8 MHz? Show your calculation.

- In Normal mode, when is TOVx set to high?

- In CTC mode, when is OCFx set to high?