### OBJECTIVES:

- To write a program to convert binary to ASCII on Dragon12+ Board.

### REFERENCE:

- Mazidi and Causey “HCS12 Microcontroller and Embedded Systems,” Chapters 4 and 6.

### MATERIALS:

- CodeWarrior

- Dragon12+ Trainer board

On Dragon12 plus board all the PORTB bits are connected to LEDs. They are controlled by the PTJ1 pin. The LEDs will not show the data on PORTB unless you turn off the PTJ1 bit. To do that, we must make PTJ=output (DDRJ=0xFF) and PTJ1 =0 (logic zero).   Also the 8 DIP switches are connected to PTH. See page 24 of Dragon12 plus User’s manual.

### ACTIVITY 1

Write a program to get a byte of binary data from PTH switches (00-0xFF) and after converting it to ASCII display the result on LEDs of PORTB (example 0xFF becomes 323535). Place a 2-second delay in between each of three ASCII showings so you can examine the result.  Run the program to verify it works.

### ACTIVITY 2

Write a program to convert the following ASCII to binary and display the result on LEDs of PORTB.

ASCII_1	EQU	$31		; Least significant digit

ASCII_2	EQU	$39		; Mid digit

ASCII_3	EQU	$35		; Most significant digit

Change the above number and verify the result.

Name:						Date: 				Class:

- Find the value in register A after the following code.

CLC

LDAA #$94

RORA

RORA

RORA

RORA

A =		in hex

- Find the value in A, the accumulator, after the following code.

CLC

LDAA #$94

ROLC

ROLC

ROLC

ROLC

A =		in hex

3. Show the code for swapping the nibbles in register B.

4. Can the ROR instruction work on any register?

5. Find the value on PORTB after the following code.

LDAA #$AA

EORA #$FF

STAA	PORTB

PORTB =       			in binary

6. Find the value in PORTB after the following code.

LDAA #$55

EORA #$FF

STAA PORTB

PORTB = 			in binary