### OBJECTIVE:

- To interface RTC to HCS12.

### REFERENCES:

- Mazidi and Causey, “HCS12 Microcontroller and Embedded Systems,” Chapter 16.

- www.maxim-ic.com

### MATERIALS:

- HCS12 Trainer

- CodeWarrior

- RTC module

### ACTIVITY 1

Connect the RTC module to the HCS12 Trainer or set up RTC circuit as shown in Chapter 16. Write a program to display time and date on PC screen or an LCD.

### ACTIVITY 2

Connect a buzzer to the RTC. After finishing ACTIVITY 1, write a program to sound the buzzer every 25 seconds past the minute.  The Buzzer should stop after 10 seconds

Name:						Date: 				Class:

1.  In connecting the RTC, indicate the direction of pins SDI, SDO, SCLK, and CE from the         point of view of the HCS12.

2.  Give the steps for setting the time and date in RTC.

3.  Give the steps for reading the time and date in RTC.

4.  In the RTC used in this Lab give the address for the following.

(a) Seconds

(b) Minutes

(c) Hours

(d) Day of the week

(e) Day of the Month

(f) Month

(g) Year