### OBJECTIVE:

- To access EEPROM of HCS12

### REFERENCES:

- Mazidi and Causey “HCS12 Microcontroller and Embedded Systems,” Chapter 14

### MATERIALS:

- HCS12 Trainer

- CodeWarrior

### ACTIVITY 1

Connect the HCS12 Trainer to the Serial Port of the x86 IBM PC and a Switch to a pin on one of the PORTs. Open a Hyper Terminal session. Write and test a program with the following components:

1) Your program writes the names (last name and first name) to the EEPROM. The names will be sent to the HyperTerminal on PC according the status of SW.

- if SW=0,  your first name will be displayed on PC screen.

3)  if SW=1, your last name will be displayed on PC screen.

Your program must store the names on the EEPROM before they are sent to the HyperTerminal on PC.

### ACTIVITY 2

Connect the HCS12 Trainer to the Serial Port of the x86 IBM PC and a Switch to a pin on one of the PORTs. Write and test a program with the following components:

- if SW=0 we will have the following:

  - the HCS12 Trainer sends a message to the PC’s Hyperterminal asking the user for a string of characters to be typed.

  - The program gets the characters from x86 PC keyboard and saves them in an internal buffer in some RAM area.

  - The HCS12 will write the contents of the RAM buffer to EEPROM of the HCS12.

2) if SW=1, the program displays the data saved in EEPROM on x86 PC screen.

Name:						Date: 				Class:

1.  Give the following information for EEPROM used in the HCS12 Trainer used in this Lab.

Capacity:				Organization:

2.  In the HCS12, give the registers used for writing data to EEPROM.  Explain the role each register plays in the process.

3.  In the HCS12, give all the registers used for reading data from EEPROM. Explain the role each register plays in the process.

4) Compare and contrast the Flash and EEPROM memories in the HCS12 microcontroller.

5) Compare and contrast the process of writing to the Flash and EEPROM memories in the HCS12 microcontroller.

6) Compare and contrast the process of reading from the Flash and EEPROM memories in the HCS12 microcontroller.