### OBJECTIVE:

- To interface an LM35 (or LM34) temperature sensor to HCS12.

### REFERENCES:

- Mazidi and Causey, “HCS12 Microcontroller and Embedded Systems,” Chapter 13.

- For the sensor's data sheet, see National Semiconductor’s web site: www.national.com

### MATERIALS:

- HCS12 Trainer

- CodeWarrior

- LM35 (or LM34)

You need to complete Lab 5 to experiment with the temperature sensors before performing this lab.

### ACTIVITY 1

In place of a potentiometer in Lab5, use the LM34 sensor and set up the circuit shown in Chapter 13 of the textbook. Then write a program to display the temperature on the LCD (or PC monitor) continuously.  Heat up or cool down the sensor and observe the temperature.

Name:						Date: 				Class:

1.  What is a transducer?

2.  What is the form of the transducer output?

3.  What is preprocessing of transducer signals to be fed into an ATC called?

4.  The LM35 and LM34 produce a _______ mV output for every degree of change in

temperature.

5.  The LM35/LM34 is a ____________ (linear, nonlinear) device. Discuss the advantages of

linear devices and of nonlinear devices.

6.  Explain signal conditioning and its role in data acquisition.