### OBJECTIVES:

- To examine and use CodeWarrior assembler.

- To examine and use CodeWarrior simulator.

### REFERENCE:

- Mazidi and Causey “HCS12 Microcontroller and Embedded Systems,” Chapter 2.

### MATERIALS:

- Download CodeWarrior assembler/simulator and its Tutorial from the following websites:

### WEB SITES:

www.freescale.com		for CodeWarrior

www.MicroDigitalEd.com	for Codewarrior Tutorial

### ACTIVITY 1

Write and assemble a program to move value 99H to register A. Then from register A move it to RAM locations  1220H – 1227H. Use the CodeWarrior simulator to single-step the program and examine the RAM locations.

### ACTIVITY 2

Write and assemble a program to add all the single digits of your ID number and save the result in register A. Pick 7 random numbers (all single digit) if you do not want to use your ID number. Then use the CodeWarrior simulator to single-step the program and examine the registers.

Name:						Date: 				Class:

1.  Indicate the size (8- or 16-bit) of each of the following registers.

PC =			A=			B =

X =			Y =			PORTB=

2. For Question 1, indicate the largest value (in decimal) that each register can contain.

PC =				A =			B =

X =				Y=			PORTB=

3. For Question 1, indicate the largest value (in hex) that each register can contain.

PC =				A =			B =

X =				Y=			PORTB=

4. Who generates each of the following files and what is the use of each.

.asm

.lst

.obj

.s19