### OBJECTIVES:

- To examine the I/O port operation using a CodeWarrior simulator.

- To trace through a JSR subroutine using a MPLAB simulator.

### REFERENCE:

- Mazidi and Causey “HCS12 Microcontroller and Embedded Systems,” Chapters 3 and 4.

### MATERIALS:

- CodeWarrior assembler and simulator.

### ACTIVITY 1

Write and assemble a program to toggle all the bits of PORTB, PORTA, and PORTH continuously by sending 55H and AAH to these ports. Put a time delay between the "on" and "off" states.  Then using the simulator, single-step through the program and examine the ports. Do not single-step through the time delay call.

### ACTIVITY 2

Using a simulator, single-step the program in Activity 1. Single-step through the delay subroutine. Examine the registers of the delay subroutine and make the delay shorter or longer by changing the register values.

### ACTIVITY 3

Using a simulator, write a program to get a byte of data from PORTH and send it to PORTA. Also, give a copy of it to RAM locations 1220H and 1222H. Place a short delay between inputting the data from the port and sending it out to PORT.  Single-step the program and examine the ports and registers.

Name:						Date: 				Class:

1. Upon reset, all the ports of the HCS12 are configured as ______ (input, output).

2. To make all the bits of a port an input port we must write ____ hex to register______.

3. Which ports of the HCS12 are bits addressable?

4. What does it mean for port to be "read-modify-write"?

5. Write a program to monitor PB4 continuously. When it becomes low, it sends 55H to PORTA.

6. The PC (program counter) in HCS12 is _______-bits wide.

7.  Explain the difference between the following two instructions:

JMP

BRA

8.  Explain the difference between the following two instructions:

JSR

CALL

9. The D register in HCS12 is _______-bits wide. Give the name of each byte.