### OBJECTIVES:

- To code a program to transfer data from code ROM space into RAM locations.

- To code a program to transfer data from RAM locations to other RAM locations.

- To experiment with a look-up table.

### REFERENCE:

- Mazidi and Causey “HCS12 Microcontroller and Embedded Systems,” Chapter   6.

### MATERIALS:

- CodeWarrior assembler and simulator.

### ACTIVITY 1

Write a program to transfer a string of data from code space starting at address $8400 to RAM locations inside the CPU starting at $1200. The data representing your last name and first name is as shown below:

ORG $8400

MYDATA:	DCB	"Alex Young",0

Using the simulator, single-step through the program and examine the data transfer and registers.

### ACTIVITY 2

Add the following subroutine to the program in Activity 1, single-step through the subroutine and examine the RAM locations.

After data has been transferred from ROM space into RAM, this subroutine should copy the data to RAM locations starting at $1360.

### ACTIVITY 3

Write a program to calculate y where y = x2 + 9x + 7.   x value is between 0 and 9 and the look-up table for x2 is located at the address (code space) of 8400H. Register A has the x, and at the end of the program PORTB should have y. Use the simulator to change the x value and single-step through the program, examining the registers as you go.

Name:						Date: 				Class:

1.  Explain the difference between the following two instructions:

LDAA $12

LDAA #$12

2.  Circle the invalid instructions.

LDAA #$5F			LDAA #$2FF

MOV 1,Y,1,X			LDAA 1,X

3.  Explain the difference between the following two instructions:

LDAA 1,X

LDAA 1,X+

4. Write a short program to clear RAM locations 1500H through 1550H

5. In generating a square of numbers between 0 and 9 there are two methods: (a) using the         MUL instruction, or (b) using a look-up table. Which one is faster in the HCS12?