### OBJECTIVE:

- To write a program to perform ASCII and BCD conversion.

### REFERENCE:

- Mazidi and Causey “HCS12 Microcontroller and Embedded Systems,” Chapter 6 or 7.

### MATERIALS:

- CodeWarrior.

### ACTIVITY 1

Write a program to convert the following ASCII data to packed BCD.

ORG	$8120

MYDATA:	FC.B	'1998'

Place the packed BCD result in some RAM locations. Using CodeWarrior simulator, single-step the program and examine the data.

### ACTIVITY 2

Write a program to convert the following BCD data to ASCII. Place the result in some RAM locations. The BCD data is stored in ROM starting at $8150. The data is stored as follows:

ORG	$8150

MYDATA:	FC.B	$42,$58,$64,$29		;pick your own data.

Using CodeWarrior simulator, single-step the program and examine the data.

### ACTIVITY 3

Write a program to find the average of the following numbers. Notice that they are ASCII and must be converted to BCD before the average is calculated.

ORG $8190

MYDATA:	FC.B	'8','9','4','7','6','2','5'	;pick your own data

Notice that you must first bring the data from ROM space into the CPU's RAM, then add them together. Using CodeWarrior simulator, single-step the program and examine the data.

Name:						Date: 				Class:

- Find the value in register A after the following code.

CLC

LDAA #$45

RORA

RORA

RORA

A =		in hex

- Find the value in A, the accumulator, after the following code.

CLC

LDAA #$45

ROLC

ROLC

ROLC

A =		in hex

3. Show the code for swapping the nibbles in register A.

4. Can the ROL instruction work on any register?

5. Find the value in register A register after the following code.

LDAA #0

EORA #$FF

A =       	in hex

6. Find the value in register A after the following code.

LDAA #$55

EORA #$FF

A = 		in hex