### OBJECTIVES:

- To write a program to transfer data to x86 PC serially.

### REFERENCES:

- Mazidi and Causey, “HCS12 Microcontroller and Embedded Systems,” Chapter 10.

### MATERIALS:

- CodeWarrior

- Project Board with HCS12 MCU Module

### ACTIVITY 1

Download and run the Sending data via Serial COM port of Project Board

www.MicroDigitalEd.com/HCS12/

On the PC side you need to use the HyperTerminal program. Here are the steps to set up HyperTerminal:

1. In Microsoft Windows Accessories, click on HyperTerminal (if you get a modem installation

option, choose "NO").

2. Type a dummy name, and click OK.

3. For "Connect Using" select COM1 and click OK.

4. Pick 9600 (or whatever baud rate you programmed the HCS12 Trainer), 8 bit data, no parity bit, and 1 stop bit.

5. Change the "Flow Control" to NONE or Xon/Xoff and click OK (definitely do not choose

the hardware option).

After making sure that it works, modify it to transfer your last name to the PC continuously.  You should see the x86 PC screen is filled with your name. Now, modify the program and place a space character at the end of your name and see the difference.  If you are using Windows Vista or Windows 7, then you can use Tara Terminal. See www.MicroDigitalEd.com for a tutorial on Tara Terminal.

### ACTIVITY 2

Modify and run the program in Activity 1 to place line feed and carriage return at the end of your name.

### ACTIVITY 3

Using a DIP switch of PORTB, modify the program in Activity 2 to transfer either your last name or your first name. For example, if PORTB7=0, then it transfers your first name and if PORTB7=1 your last name is sent out serially.

Name:						Date: 				Class:

1. The HCS12 TxD and RxD signals ________ (are, are not) TTL-compatible.

2. In this lab, what is the role of the MAX233 (MAX232) chip?

3. With XTAL=4 MHz, what is the maximum baud rate for the HCS12?

4. Show how to achieve the maximum baud rate in Question 3.

5. What is the role of TDRE and RDRF?   State to which register they belong to.

6. True or false. The HCS12 transfers data in full-duplex.

7. For full duplex, what are the absolute minimum signals needed between the HCS12 and the

x86 PC?  Give their names.