### OBJECTIVES:

- To program the RTI (real Time Interrupt) interrupt of the HCS12.

### REFERENCE:

- Mazidi and Causey, “HCS12 Microcontroller and Embedded Systems,” Chapter 11.

### MATERIALS:

- Project Board with HCS12 MCU Module

- CodeWarrior

### ACTIVITY 1

Download the R RTI Interrupt for project Board   from the www.MicroDigitalEd.com site.

This program toggles the PT0 (buzzer) continuously while Real Timer Interrupt (RTI) of the HCS12 will toggle an LED connected to PORTA every so often.  a) Modify the ISR program to flash (toggle) the LED every 2 sec.

### ACTIVITY 2

In the main program of Activity 1 we set the RTI parameters. Use the PORTB DIP switches to set the RTI parameters and experiment with rate of LED flashing.

### ACTIVITY 3 (Advanced)

Modify the program in Activity to toggle an LED of PORTA and use RTI to sound the buzzer on PT0.  In other words, the roles are reversed. Again use PORTB DIP switches to set the RTI parameters and create different sounds on the buzzer.

Name:						Date: 				Class:

1. True or False. The RTI uses the frequency 1/2 of XTAL for its timing.

2. True or False. Upon reset, the highest priority is assigned to RTI interrupts.

3. True or False. Upon Reset. All the peripheral interrupts are masked

4. In HCS12, indicate how you enable the peripheral interrupts globally.

5. Which register is used to set the RTI rate?

6.  Explain the role of CLI instruction in the main program of Activity 1.