### OBJECTIVES:

- To examine the flag bits of status register.

### REFERENCE:

- Mazidi and Causey “HCS12 Microcontroller and Embedded Systems,” Chapter 2.

### MATERIALS:

- CodeWarrior assembler and simulator.

### ACTIVITY 1

Open a project in CodeWarrior, click on main.asm and after deleting everything, copy and paste the shell from the www.MicroDigitalEd.com website or from Lab 2A. Write and assemble a program to add the following data and then use the simulator to single step program and examine the C, H and Z flags after the execution of each addition.

$99, 01, $66, $89, $97, $35

### ACTIVITY 2

Write and assemble a program to load a value into each RAM location of $1220 – $1224. Use the COM instruction to complement the value in each location. Use the simulator to single-step and examine the flags and RAM content after the execution of each instruction.

Name:						Date: 				Class:

1. Find the value of the C, H, and Z flags after the execution of  the following code.

LDAA #$85

ADDA #$9F

2. Find the value of the C, H, and Z flags after the execution of  the following code.

LDAA #$99

ADDA #$67

ADDA #$FF

3. Find the value of the C, H, and Z flags after the execution of  the following code.

LDAA  #$AA

ADDA #$55

4. Find the value of the C, H, and Z flags after the execution of  the following code.

LDAA #$FE

ADDA #1

ADDA #1

5. Upon reset, what is the value in the PC register?

6. Give the size of D,X, and Y registers.

6. True or false. D is physically a different register from registers A and B.