### OBJECTIVES:

- To write a program to convert packed BCD to ASCII on Project Board.

- To write a program to convert ASCII to packed BCD on Project Board.

### REFERENCE:

- Mazidi and Causey “HCS12 Microcontroller and Embedded Systems,” Chapters 4 and 5.

### MATERIALS:

- CodeWarrior

- Project Board and HCS12 MCU Module

### ACTIVITY 1

Write a program to get a byte of packed BCD from PORTB switches and display the result on LEDs of PORTA with 2-second delay in between each ASCII showing. Run the program to verify it works.

### ACTIVITY 2

Modify the program in Activity 1 to sound the buzzer if the input value on DIP switches are not packed BCD ($00-$99). That means, it will convert only the valid packed BCD data and displays ASCII numbers on LEDs and if the input data is not valid packed BCD (e.g. 10011111 binary) it will sound the buzzer and LEDs will have all zeros.

### ACTIVITY 3

Write a program to convert two ASCII numbers to a packed BCD and display the BCD result on LEDs of PORTA. Run the program to verify it works.

ASCII_1	EQU	$39

ASCII_2	EQU    $37

Name:						Date: 				Class:

- Find the value in register A after the following code.

CLC

LDAA #$45

RORA

RORA

RORA

A =		in hex

- Find the value in A, the accumulator, after the following code.

CLC

LDAA #$45

ROLC

ROLC

ROLC

A =		in hex

3. Show the code for swapping the nibbles in register A.

4. Can the ROL instruction work on any register?

5. Find the value in register A register after the following code.

LDAA #0

EORA #$FF

A =       	in hex

6. Find the value in register A after the following code.

LDAA #$55

EORA #$FF

A = 		in hex