### OBJECTIVES:

- To compile, download and run a C program using CodeWarrior C compiler and Project Board.

### REFERENCE:

- Mazidi and Causey “HCS12 Microcontroller and Embedded Systems,” Chapter 7.

### MATERIALS:

- CodeWarrior (C compiler)

- Project Board with HCS12 MCU Module

Insert the HCS12 MCU Module into the Project Board and connect the power as discussed on the Toggling LEDs of PORTA In Project Board program given on the www.MicroDigitalEd.com site under Project Board Sample programs.

On Project Board, 4 pins of PORTA are connected to LEDs permanently. We need to connect the other 4 pins as discussed on  Toggling LEDs of PORTA in Project Board in C Sample Program

Open the CodeWarrior Project for a C language and copy and paste Toggling LEDs of PORTA in Project Board in C Use these steps to run it.

- Make sure HCS12 is in BDML mode before downloading into the board.

- Press F7 to make

- Press F5 to download

- Press F5 once more (or green arrow) to start to run the program.

- Now, examine the LEDs of PORTA on Project Board to see them toggle.

f)   Keep the LEDs connected to PORTA for future Labs.

### ACTIVITY 2

Do not change the values in the body of MSDelay function. However, when calling MSDelay(n); C function, change the value of n to make the time delay shorter (or longer) and run the program to see the LEDs toggle faster (or slower). Run and verify it.

### ACTIVITY 3

If you have access to digital Scope, measure the size of MSDelay in msec.

Name:						Date: 				Class:

1. To create a loop of 10,000 iterations we need data type of (unsigned char, unsigned int).

2. To create a loop of 200 iterations we need data type of (unsigned char, unsigned int).

3. What is the opposite of 0x55?

4. What is the opposite of 0xAA?

5. True or false. For the CodeWarrior C language syntax we can use 0b01010101 for binary in place of 0x55 in hex.

6.  To make PORTB an output port, we must write ______to register_______.

7. On the Project Board, the LEDs are connected to the port_______ permanently.

8. In the toggle program reduce the size of the delay to point in which the LEDs looks to be on all the time (no longer toggling) . How many millisecond is that?